|  | CORE SWITCH | OST | Extreme | I-tek | Door Sanchar | Fore | GNDEC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Switch architecture should be modular / virtual chassis based. |  |  |  |  |  |  |
| 2 | Switch should have internal redundant power supplies and fans. |  |  | To remove the redundant fan tray |  |  | Need to discus why??? |
| 3 | Switch should have wire-speed, non-blocking and distributed forwarding on all the ports. |  |  |  |  |  |  |
| 4 | The Core Switch should have the following ports from day one in any combination: |  |  |  |  |  |  |
|  | · 12x 10G SFP+ or more |  |  | Mention type & Qty of media |  |  | Single Mode Qty. as per proposed design |
|  | · 20x 1G SFP or more |  |  | Mention type & Qty of media |  |  | Single Mode Qty. as per proposed design |
|  | · 20x 10/100/1000 BaseT or more |  |  |  |  |  |  |
|  | · 6x 40G QSFP or more |  |  | Mention type & Qty of media |  |  | Single Mode Qty. as per proposed design |
| 5 | Should support minimum 800 Gbps or more backplane capacity. |  |  |  |  |  |  |
| 6 | Should support minimum forwarding rate 595 Mpps or more |  |  |  |  |  |  |
| 7 | Support for IPv4 and IPv6 from day one. |  |  |  |  |  |  |
| 8 | Should support Q-in-Q VLAN tagging or equivalent feature. |  |  |  |  |  |  |
| 9 | Should have Static Routing, RIP, OSPF, VRRP, ISIS and PIM from day one. | ISIS is required at ISP level | ISIS and OSPF is required at ISP level | To add 802.1X protocol support for authentication ???? | ISIS is required at ISP level | ISIS is required at ISP level | GNDEC will remove ISIS. Regarding 802.1X We will see. This is an addition |
| 10 | Should support PVST / PVST+ or equivalent spanning tree protocol |  |  |  |  |  |  |
| 11 | Should support Hot Standby Router Protocol (HSRP) or equivalent protocol to create redundant topologies. |  |  |  |  |  |  |
| 12 | Should support Unidirectional Link Detection Protocol (UDLD) or equivalent protocol to allow unidirectional links failure detection |  |  |  |  |  |  |
| 13 | Should support minimum 8 ports or more of Bandwidth aggregation through Ether Channel or equivalent protocol enhances fault tolerance and offers higher-speed aggregated bandwidth |  |  |  |  |  |  |
| 14 | Should support VTP or equivalent protocol for dynamic VLAN registration |  |  |  |  |  |  |
| 15 | Should support min 28K MAC addresses and min 4096 active VLANs. |  |  |  | Should support 512 K MAC |  | Minimum requirement is floated |
| 16 | Support management using CLI, GUI, using Web interface. Additionally, management can also be done using NMS. |  |  | To remove GUI |  |  | Can’t remove |
| 17 | Hardware and Software of the switch should be common criteria EAL / NDPP certified. |  |  |  |  |  |  |
| 18 | Should support MPLS and ISIS | MPLS and IS-IS is required for ISPs | MPLS and IS-IS is required for ISPs |  | MPLS and IS-IS is required for ISPs | MPLS and IS-IS is required for ISPs | GNDEC will remove MPLS and ISIS |
| 19 | Should be quoted with 3 years direct OEM TAC support and next business day hardware shipment. |  |  |  |  |  |  |