| SN | Access Point - B | OST | Extreme | I-tek | Door Sanchar | Fore | GNDEC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | The AP shall be indoor type and can be mounted on wall / ceiling. Required mounting kit should be included with each AP. |  |  |  |  |  |  |
| 2 | The AP shall have at least one Gigabit Ethernet port supporting 10/100/1000BaseT with 802.3af/at PoE support, 3 port 10/100 including 1 port PoE out to power VOIP phone and One Digital PBX Pass through port support. | Specific to some vendors | Specific to some vendors |  |  |  | We need such type of device. It can be discussed. Generally good companies have this |
| 3. | AP shall support 802.11n standard along with legacy 802.11a/b/g standards. |  |  |  |  |  |  |
| 4. | AP shall have Dual radio to operate in 2.4 GHz and 5 GHz band simultaneously. |  |  |  |  |  |  |
| 5. | AP shall have integrated / external antenna with minimum 2x2 MIMO with 2 spatial streams. |  |  |  |  |  |  |
| 6. | AP shall give 600 Mbps per AP (300 Mbps / Radio) or higher throughput. |  |  |  |  |  |  |
| 7. | The AP shall have auto channel selection feature to avoid interference. |  |  | This feature is supported by controller |  |  | It should do this in sync with controller |
| 8. | The AP shall support indoor wireless mesh configuration. |  |  | Remove meshing from in room AP |  |  | Can be thought of |
| 9. | AP shall provide at least 8 BSSID's per radio. |  |  |  |  |  |  |
| 10. | Transmit power shall be configurable as per India specific norms for indoor deployment and shall be adjustable within the permissible range. |  |  |  |  |  |  |
| 11. | AP shall be able to optimize capacity and performance using airtime fairness feature. |  |  |  |  |  |  |
| 12. | AP shall provide Radio transmit power of at least 19 dBm for 2.4 GHz and 16 dBm for 5 GHz bands. (Max power should be limited as per Govt EIRP limit for indoor) |  |  | Change 20 dBm for 2.4Ghz and 18dBM for 5 Ghz |  |  | Already quoted less as per govt. norms. Higher vendor can quote within EIRP Limit |
| 13 | 802.1Q based VLANs to be supported. |  |  |  |  |  |  |
| 14 | AP should provide L2TP/PPoE or equivalent tunnelling support. |  |  | System should provide L2TP/PPoE or equivalent tunnelling support. |  |  | GNDEC will Write “AP/System should provide L2TP/PPoE or equivalent tunnelling support.” It should do this in sync with controller |
| 15 | AP shall support 802.11e based QoS features. |  |  |  |  |  |  |
| 16 | AP shall provide software queues for prioritization of latency prone traffic like voice and video. |  |  |  |  |  |  |
| 17 | AP shall support QoS and/or VLAN based traffic classification. |  |  | System shall support ToS and/or VLAN based traffic classification. |  |  | GNDEC will write AP/System. It should do this in sync with controller |
| 18 | Support for Spectra link Voice or WMM with UAPSD |  |  |  |  |  |  |
| 19 | AP shall support per user or per WLAN based rate limiting. |  |  | Remove the clause |  |  | GNDEC will delete this |
| 20 | AP shall have WISPr support for walled garden deployments as well as provisioning Wi-Fi hotspots with time based user access/session control. |  |  | Remove the clause |  |  | GNDEC will delete this |
| 21 | AP shall support WEP, WPA2-AES, WPA-PSK, WPA-TKIP. |  |  |  |  |  |  |
| 22 | AP shall support 802.11i, 802.1x based authentication |  |  |  |  |  |  |
| 23 | AP shall support Captive portal for guest user authentication. |  |  | System shall support Captive portal for guest user authentication |  |  | OK. It should do this in sync with controller |
| 24 | AP shall support centralized authentication with external RADIUS, LDAP or Active Directory. |  |  |  |  |  |  |
| 25 | Operating Temperature: 0 to 50 degree Centigrade. |  | Operating temperature too high. Do it 0-40 degree centigrade | 0-45 degree centigrade |  |  | Ok no issues as this is indoor |
| 26. | Operating Humidity: 15% - 95% noncondensing. |  |  |  | 15% -90% |  | Can be done |
| 27 | IPv4, IPv6, dual-stack Should be supported |  |  |  |  |  |  |