|  |  | ISOLATED FOOTING DESIGN |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FORCES DETAILS : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Axial Load = P1 = |  |  |  |  |  | 21 |  |  | T |  |  |  |  |
| Moment along Major axis = Mx = |  |  |  |  |  | 0.50 |  |  | T-m |  |  |  |  |
| Moment along Minor axis = Mz = |  |  |  |  |  | 0.5 |  |  | T-m |  |  |  |  |
| COLUMN DETAILS : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length = ( Larger Dimension of Column ) |  |  |  |  |  | 0.23 |  |  | m |  |  |  |  |
| Width = ( Smaller Dimension of Column ) |  |  |  |  |  | 0.23 |  |  | m |  |  |  |  |
| FOOTING DETAILS : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Length = L = |  |  |  |  |  | 1.75 |  |  | m |  |  |  |  |
| Width = B = |  |  |  |  |  | 1.75 |  |  | m |  |  |  |  |
| Depth = D = |  |  |  |  |  | 0.3 |  |  | m |  |  |  |  |
| Cover to Main R/f = |  |  |  |  |  | 0.05 |  |  | m |  |  |  |  |
| Diameter of R/f in Footing = |  |  |  |  |  | 12 |  |  | mm |  |  |  |  |
| SOIL DETAILS : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Density of Soil = = |  |  |  |  |  | 1.7 |  |  | T/m3 |  |  |  |  |
| Depth of foundation below G.L = H = |  |  |  |  |  | 1.0 |  |  | m |  |  |  |  |
| MATERIAL CONSTANT : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concrete Grade = |  | M - 25 |  | N/mm2 |  |  |  |  |  |  |  |  |  |
| R/f Grade = |  | Fe -500 |  | N/mm2 |  |  |  |  |  |  |  |  |  |
| Load Factor = f = |  | 1.5 |  |  |  |  |  |  |  |  |  |  |  |
| DESIGN FORCES : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Axial load = P = |  |  |  |  |  | 21.00 |  |  | T |  |  |  |  |
| Self Weight of Footing = |  |  |  |  |  | 2.30 |  |  | T |  |  |  |  |
| Total Vertical Load = |  |  |  |  |  | 23.30 |  |  | T |  |  |  |  |
| Moment along Major axis |  |  |  |  |  | 0.5 |  |  | T-m |  |  |  |  |
| Moment along Minor axis |  |  |  |  |  | 0.5 |  |  | T-m |  |  |  |  |
| PRESSURE CALCULATION : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SBC = |  |  |  |  |  | 8.6 |  |  | T/m2 |  |  |  |  |
| FOR EARTHQUAKE CONDITION ALLOWABLE S.B.C = |  |  |  |  |  |  |  |  | 1.25 x | 8.6 = | 10.75 | T/m2 |  |
| P = | P | + | Mx |  |  | + |  |  | Mz |  |  |  |  |
|  | A | - | Zx |  |  | - |  |  | Zz |  |  |  |  |
| Pmax = | 7.61 + | 0.56 + | 0.5598 = |  |  | 8.73 |  |  | T/m2 | < S.B.C |  | SAFE |  |
| Pmin | 7.61 - | 0.56 - | 0.5598 = |  |  | 6.49 |  |  | T/m2 | TENSION NOT CREATED |  |  |  |
| R/F CALCULATION FROM BENDING CONSIDERATION : |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 1750 |  |  |  |  |  |  |  |  |  |  |
|  |  | 760 |  |  |  |  |  |  |  |  |  |  |  |
| 1750 |  | 230 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 230 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 760 |  |  |  |  |
| 6.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T/M2 |  |  |  |  |  |  |  |  |  |  | 8.7 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | T/M2 |  |  |
| MOMENT AT CRITICAL SECTION ALONG WIDTH : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M = | Pmax x 0.76 x 0.76 / 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| M = | 2.52 | T-m |  |  |  |  |  |  |  |  |  |  |  |
| Mu = | 37.80 | KN-m |  |  |  |  |  |  |  |  |  |  |  |
| Ast = | 431.39 | mm2 | ( To be distributed in 1.75 m Length ) |  |  |  |  |  |  |  |  |  |  |
| Min Req = | 360 | mm2 | ( Ast = 0.12 x b x D ) |  |  |  |  |  |  |  |  |  |  |
| Required Spacing of Bar = 12 # @ 260 C/C ALONG WIDTH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROVIDE 12 # @ 200 C/C ALONG WIDTH |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  Ast = | 753.98 | mm2 |  |  |  |  |  |  |  |  |  |  |  |
| MOMENT AT CRITICAL SECTION ALONG LENGTH : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M = | Pmax x 0.76 x 0.76 / 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| M = | 2.52 | T-m |  |  |  |  |  |  |  |  |  |  |  |
| Mu = | 37.80 | KN-m |  |  |  |  |  |  |  |  |  |  |  |
| Ast = | 431.39 | mm2 | ( To be distributed in 1.75 m Length ) |  |  |  |  |  |  |  |  |  |  |
| Min Req = | 360 | mm2 | ( Ast = 0.12 x b x D ) |  |  |  |  |  |  |  |  |  |  |
| PROVIDE 12 # @ 200 C/C ALONG LENGTH |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  Ast = | 434.99 | mm2 |  |  |  |  |  |  |  |  |  |  |  |
| CHECK FOR ONE WAY SHEAR ALONG LENGTH : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Critical Section is at distance of ' d ' from face of column |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 0.76 |  |  |  |  |  |  |  |  |  |  |  |
|  | 1.75 | 0.76 |  |  |  | 0.51 |  |  |  |  |  |  |  |
|  |  | 1.75 |  |  |  |  |  |  |  |  |  |  |  |
| Shear force at critical section = |  |  | 8.73 x |  |  | 0.51 |  |  |  |  |  |  |  |
|  |  | V = | 4.45 |  |  | T |  |  |  |  |  |  |  |
|  Actual Shear stress v = |  |  | Vu |  |  |  |  |  |  |  |  |  |  |
|  |  |  | b x d |  |  |  |  |  |  |  |  |  |  |
|  |  | = | 1.5 x |  |  | 4.45 |  |  | x 10000 |  |  |  |  |
|  |  |  | 1000 x |  |  | 250 |  |  |  |  |  |  |  |
|  |  | = | 0.267 |  |  | N / mm2 |  |  |  |  |  |  |  |
| Pt = | 100 x As | = | 100 x |  |  | 434.99 |  |  | = | 0.17 |  |  |  |
|  | b x d |  | 1000 x |  |  | 250 |  |  |  |  |  |  |  |
|  c = | 0.28 | N / mm2 |  |  |  |  |  |  |  |  |  |  |  |
|  | < v |  |  |  |  |  |  |  |  |  |  |  |  |
|  | HENCE SAFE |  |  |  |  |  |  |  |  |  |  |  |  |
| CHECK FOR ONE WAY SHEAR ALONG WIDTH : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Critical Section is at diatance of ' d ' from face of column |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shear force at critical section = |  |  | 8.73 x |  |  | 0.51 |  |  |  |  |  |  |  |
|  |  | V = | 4.45 |  |  | T |  |  |  |  |  |  |  |
|  Actual Shear stress v = |  |  | Vu |  |  |  |  |  |  |  |  |  |  |
|  |  |  | b x d |  |  |  |  |  |  |  |  |  |  |
|  |  | = | 1.5 x |  |  | 4.45 |  |  | x 10000 |  |  |  |  |
|  |  |  | 1000 x |  |  | 250 |  |  |  |  |  |  |  |
|  |  | = | 0.267 |  |  | N / mm2 |  |  |  |  |  |  |  |
| Pt = | 100 x As | = | 100 x |  |  | 753.98 |  |  | = | 0.30 |  |  |  |
|  | b x d |  | 1000 x |  |  | 250 |  |  |  |  |  |  |  |
|  c = | 0.31 | N / mm2 |  |  |  |  |  |  |  |  |  |  |  |
|  | HENCE SAFE |  |  |  |  |  |  |  |  |  |  |  |  |
| CHECK FOR TWO WAY SHEAR : |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Critical Section is at distance of 'd/2' from face of column |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 1.75 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 0.48 |  |  |  |  |  |  |  |
|  | 1.75 |  | 0.48 |  |  |  |  |  | d/2 |  |  |  |  |
|  |  |  |  |  |  | d/2 |  |  |  |  |  |  |  |
| Shear force at Critical Section = |  |  | 8.73 x |  |  | Area of Shadeded Portion |  |  |  |  |  |  |  |
|  |  | = | 8.73 x |  |  | 2.8321 |  |  |  |  |  |  |  |
|  |  | = | 24.71 T |  |  |  |  |  |  |  |  |  |  |
| Critical Perimeter = |  | 2 x | 0.48 |  |  | + 2 x |  |  | 0.48 |  |  |  |  |
|  | = | 1.92 m |  |  |  |  |  |  |  |  |  |  |  |
|  Actual Shear stress v = |  |  | Vu |  |  |  |  |  |  |  |  |  |  |
|  |  |  | b x d |  |  |  |  |  |  |  |  |  |  |
|  |  | = | 1.5 x |  |  | 24.71 |  |  | x 10000 |  |  |  |  |
|  |  |  | 1920.00 x |  |  | 250 |  |  |  |  |  |  |  |
|  |  | = | 0.772 |  |  | N / mm2 |  |  |  |  |  |  |  |
| Allowable Shear stress = ks.c |  |  |  |  |  |  |  |  | ( CLAUSE 31.3.3 IS 456 : 2000 ) |  |  |  |  |
| where, |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ks = | ( 0.5+ ) |  |  |  |  |  |  |  |  |  |  |  |  |
|  = | Short Side of Column = 0.23. |  |  |  |  | = 1.00 |  |  |  |  |  |  |  |
|  | Long Side of Column = 0.23 |  |  |  |  |  |  |  |  |  |  |  |  |
| ks = | 1.50 | but not greater than 1.0 |  |  |  |  |  |  |  |  |  |  |  |
| Where, |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c = 0.25  fck |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c = 0.25  | M - 25 | = |  |  | 1.25 |  |  | N/mm2 |  |  |  |  |
| Allowable Shear stress = ks.c = |  |  | 1.25 |  |  | N/mm2 |  |  |  | SAFE |  |  |  |
| CHECK FOR OVERTURNING : |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ALONG LENGTH : |  |  |  |  |  |  |  |
|  |  | Rm |  |  |  | Restoring Moment = |  |  |  | 21.00 | x 0.88 |  |  |
|  |  |  |  |  |  |  |  |  | Rm = | 18.38 | T.m |  |  |
|  |  |  |  |  |  | Applied Moment = |  |  |  | 0.5 | T.m |  |  |
|  |  |  |  |  |  | F.O.S. = |  |  | Restoring Moment |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Applied Moment |  |  |  |  |
|  |  | P |  |  |  | = |  |  | 18.38 | = | 36.75 |  |  |
|  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | > | 2.2 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | SAFE |  |  |
|  |  | 0.88 |  |  |  |  |  |  |  |  |  |  |  |